Gosud

GOSUD

DATA FORMAT TSG

Table of content

History	4

1.	Overview of the format	5

2.	Description of the thermosalinometer format	6

2.1.	Data file dimensions	6

2.2.	Global attributes: meta-data	7

2.3.	Measurements	9

2.3.1.	Variables describing installation of TSG	9

2.3.2.	Variables describing installation of Temperature sensor at intake (TINT)	10

2.3.3.	Coordinates for TSG/TINT:	11

2.3.4.	Data series 1 main TSG:	12

2.3.5.	Data series 2: ocean  Temperature from the water intake	15

2.3.6.	Data series 3 : external data	16

3.	Reference tables	18

3.1.	Reference table 1 : Thermosalinometer (TSG) type	18

3.2.	Reference table 2 : Temperature sensor at intake (TINT) type	18

3.3.	Reference table 3 : PROCESSING STATES	18

3.4.	Reference table 4 : QUALITY FLAGS	19

3.5.	Reference table 5 : Parameter code table	19

3.6.	Reference table 6 : Water sample (bottle) type	20

History

| Version | Date | Comment |
| --- | --- | --- |
| 1.0 | 09/11/2005 | F. Gaillard/L. Petit de la Villéon creation of the document based on GOSUD data management user’s manual (T. Carval) |
| 1.1 | 29/11/2005 | FG/LPdV: revision after ORE-SSS technical meeting |
| 1.2 | 01/12/2005 | T. Carval : parameter naming conventions : <PARAM>_XXX where XXX = TSG, WS, TINT |
| 1.3 | 17/01/2006 | F. Gaillard, D. Mathias, correction after producing the 2000-2004 files. |
| 1.4 | 20/11/2007 | C. Lagadec, J.Grelet, F. Gaillard : integration in processing CORIOLIS/IRD |

# Overview of the format

This document aims at defining a common format for thermo-salinometer (TSG) data. This format is meant for the data exchange and processing. It should hold both real time and delayed mode data and various levels of resolution or processing.

The data sets found in the file are:

- General information on the file, the platform and the TSG installation

- Data series from the main TSG

And, whenever available:

- Data series from the Temperature sensor at water intake

- Data from salinity sample analysis

In the case of real time data, some variables may be missing. The file is updated as new data and information come in.

For delayed mode use, the data center is expected to provide a file that corresponds to a unique installation period of a thermo-salinometer on one ship. All available information must have been entered, in particular those referring to the calibration and water sample analysis. The group in charge of the validation will complement the file with the ‘corrected’ data series.

# Description of the thermosalinometer format

## Data file dimensions

| Name | Definition | Comment |
| --- | --- | --- |
| DAYD |  | Number of recorded measurements for the main TSG series |
| DAYD_WS |  | Number of recorded measurements for water samples |
| DAYD_EXT |  | Number of recorded measurements for external data |
| NCOEF_CAL | NCOEF_CAL = 5 | Number of calibration coefficients |
| NCOEF_LIN | NCOEF_LIN = 2 | Number of drift correction coefficients (linear) |
| STRING256 STRING14 STRING4 | STRING256 = 256; STRING14 = 14; STRING4 = 4; | String dimensions. |
| N1 | N1 = 1; |  |

## Global attributes: meta-data

| Name | Definition | Comment |
| --- | --- | --- |
| CYCLE_MESURE | CYCLE_MESURE = <char value>; | Cruise name or travel number Example : EGEE6 or PAST0601 |
| PROJECT_NAME | PROJECT_NAME = <char value>; | Name of the project which operates the TSG line. Example : ORE-SSS |
| PLATFORM_NAME | PLATFORM_NAME = <char value>; | Ship name Example : Pourquoi Pas ? |
| SHIP_CALL_SIGN | SHIP_CALL_SIGN = <char value>; | Ship call sign . Example : FABB |
| SHIP_MMSI |  | Ship MMSI (ASN) number Example : |
| DATE_TSG | DATE_TSG = <char value>; | Date of TSG installation : YYYYMMDDHHMISS |
| TYPE_TSG |  | Described in reference table 1 Example : SBE21 |
| NUMBER_TSG | NUMBER_TSG= <char value>; | (serial number, ex: 2250) |
| DATE_TINT | DATE_TINT = <char value> | Date of TINT installation |
| TYPE_TINT | TYPE_TINT= <char value>; | Temperature sensor at intake. Described in reference table 2 Ex: SBE3 |
| NUMBER_TINT | NUMBER_TINT= <char value>; | (serial number, ex: ????) |
| DATA_TYPE | DATA_TYPE = <char value>; | This field describes the type of data contained in the file. Example : TRAJECTORY, PROFIL or TIME_SERIE |
| DATA_MODE | DATA_MODE = <char value>; | Indicates if the file contains real time or delayed mode data. R : real time data D : delayed mode data |
| SAMPLING_PERIOD | SAMPLING_PERIOD = <char value>; | Sampling period in seconds: 6 to 3600 |
| DATE_START | DATE_START = <char value>; | Date of first measurements : YYYYMMDDHHMISS |
| DATE_END | DATE_END = <char value> | Date of last measurements : YYYYMMDDHHMISS |
| SOUTH_LATX |  | South limit of measurements |
| NORTH_LATX |  | North limit of measurements |
| WEST_LONX |  | West limit of measurements |
| EAST_LONX |  | East limit of measurements |
| FORMAT_VERSION | FORMAT_VERSION = <char value>; | File format version : 1.4 for this format |
| DATE_CREATION | DATE_CREATION = <char value>; | Date and time (UTC) of creation of this file. Format : YYYYMMDDHHMISS Example : 20011229161700 : December 29th 2001 16:17:00 |
| DATE_UPDATE (optional) | DATE_UPDATE = <char value>; | Date and time (UTC) of update of this file. Format : YYYYMMDDHHMISS Example : 20011230161700 : December 30th 2001 16:17:00 |
| DATA_RESTRICTIONS | DATA_RESTRICTIONS = <char value>; | Restriction on use for these data. Example : "NONE" |
| CITATION | CITATION = <char value>; | The citation should be used for publications. Example : "These data were collected and made freely available by the International Gosud Project and the national programmes that contribute to it." |
| COMMENT | COMMENT = <char value>; |  |
| PI_NAME | PI_NAME = <char value>; | Name of the principal investigator in charge of the TSG line. Example : GENAVIR |
| DATA_CENTRE | DATA_CENTRE = <char value>; | Code for the data centre (2 char) The data centre codes are described in the reference table 3 Example : |
| DATA_ACQUISITION | DATA_ACQUISITION = <char value>; | Acquisition data centre Example : SHOM, IRD, GENAVIR, CNRS … |
| PROCESSING_CENTRE | PROCESSING_CENTRE = <char value>; | Processing data centre Example : ORE-SSS, CORIOLIS/IRD, CORIOLIS/SISMER |
| PROCESSING_STATES | PROCESSING_STATES = <char value>; | Described in reference table 3 |
| WS_TYPE | WS _TYPE = <char value> | Model of water sample bottle Default : NA Example: OSIL Described in reference table 6 |

## Measurements

### Variables describing installation of TSG

| Name | Definition | Comment |
| --- | --- | --- |
| SSPS_DEPH | float SSPS_DEPH (1); SSPS_DEPH:long_name = "Nominal depth of water intake for salinity measurement"; DEPTH :units = "meter"; SSPS_DEPH :_FillValue = 99999.f; SSPS_DEPH :valid_min = 0.f; SSPS_DEPH :valid_max = 100.f; | Ex: SSPS_DEPH = 8 |
| SSPS_DEPH_MIN | float SSPS_DEPH_MIN (1); SSPS_DEPH_MIN:long_name = "Minimum depth of water intake"; SSPS_DEPH_MIN :units = "meter"; SSPS_DEPH_MIN :_FillValue = 99999.f; SSPS_DEPH_MIN :valid_min = 0.f; SSPS_DEPH_MIN:valid_max = 100.f; | Ex: 6 |
| SSPS_DEPH_MAX | float SSPS_DEPH_MAX (1); SSPS_DEPH_MAX:long_name = "Maximum depth of water intake"; SSPS_DEPH_MAX :units = "meter"; SSPS_DEPH_MAX :_FillValue = 99999.f; SSPS_DEPH_MAX :valid_min = 0.f; SSPS_DEPH_MAX :valid_max = 100.f; | Ex: 10 |
| CNDC_CALCOEF | double CNDC_TSG_CALCOEF(NCOEF_CAL); CNDC_TSG_CALCOEF:long_name = "Conductivity calibration coefficients"; CNDC_TSG_CALCOEF:convention = "ex: ‘a’, ‘b’, ‘c’, ‘d’, ‘m’ "; CNDC_TSG_CALCOEF:date = “ex: 20011011 “ CNDC_TSG_CALCOEF :_FillValue = 99999.f; |  |
| CNDC_LINCOEF | double CNDC_TSG_LINCOEF(NCOEF_LIN); CNDC_TSG_LINCOEF:long_name = "Conductivity linear drift correction coefficients"; CNDC_TSG_LINCOEF:convention = "ex: ‘slope’, ‘offset’"; CNDC_TSG_LINCOEF:date = “ex: 20011011 “ CNDC_TSG_LINCOEF :_FillValue = 99999.f; |  |
| SSJT_CALCOEF | double SSJT_CALCOEF(NCOEF_CAL); SSJT_CALCOEF:long_name = "Temperature calibration coefficients"; SSJT_CALCOEF:convention = "ex: ‘a’, ‘b’, ‘c’, ‘d’, ‘f0’ "; SSJT_CALCOEF:date = “ex: 20011011 “ SSJT_CALCOEF :_FillValue = 99999.f; |  |
| SSJT_LINCOEF | double SSJT_LINCOEF(NCOEF_LIN); SSJT_LINCOEF:long_name = "Temperature linear drift correction coefficients"; SSJT_LINCOEF:convention = "ex: ‘slope’, ‘offset’"; SSJT_LINCOEF:date = “ex: 20011011 “ SSJT_LINCOEF :_FillValue = 99999.f; |  |

### Variables describing installation of Temperature sensor at intake (TINT)

| Name | Definition | Comment |
| --- | --- | --- |
| SSTP_DEPH | float SSTP_DEPH (1); SSTP_DEPH:long_name = "Nominal depth of water intake for Temperature measurement"; SSTP_DEPH :units = "meter"; SSTP_DEPH :_FillValue = 99999.f; SSTP_DEPH :valid_min = 0.f; SSTP_DEPH :valid_max = 100.f; | Ex: SSTP_DEPH = 8 |
| SSTP_DEPH_MIN | float SSTP_DEPH_MIN (1); SSTP_DEPH_MIN:long_name = "Minimum depth of water intake"; SSTP_DEPH_MIN :units = "meter"; SSTP_DEPH_MIN :_FillValue = 99999.f; SSTP_DEPH_MIN :valid_min = 0.f; SSTP_DEPH_MIN:valid_max = 100.f; | Ex: 6 |
| SSTP_DEPH_MAX | float SSTP_DEPH_MAX (1); SSTP_DEPH_MAX:long_name = "Maximum depth of water intake"; SSTP_DEPH_MAX :units = "meter"; SSTP_DEPH_MAX :_FillValue = 99999.f; SSTP_DEPH_MAX :valid_min = 0.f; SSTP_DEPH_MAX :valid_max = 100.f; | Ex: 10 |
| SSTP_CALCOEF | double SSTP_CALCOEF(NCOEF_CAL); SSTP_CALCOEF:long_name = "Temperature calibration coefficients"; SSTP_CALCOEF:convention = "ex: ‘a’, ‘b’, ‘c’, ‘d’, ‘f0’ "; SSTP_CALCOEF:date = “ex: 20011011 “ SSTP_CALCOEF :_FillValue = 99999.f; |  |
| SSTP_LINCOEF | double SSTP_LINCOEF(NCOEF_LIN); SSTP_LINCOEF:long_name = "Temperature linear drift correction coefficients"; SSTP_LINCOEF:convention = "ex: ‘a’, ‘b’"; SSTP_LINCOEF:date = “ex: 20011011 “ SSTP_LINCOEF :_FillValue = 99999.f; |  |

### Coordinates for TSG/TINT:

| Name | Definition | Comment |
| --- | --- | --- |
| DATE | char DATE (DAYD,STRING14); DATE:long.name = “Date of main instrument measurements” INST_REFERENCE:conventions = “yyyymmddhhmmss” | This is the original data describing the date, it must not be lost |
| DAYD | double DAYD(DAYD); DAYD:long_name = "Julian day (UTC) of each measurement"; DAYD:standard_name = "time"; DAYD:units = "days since 1950-01-01 00:00:00 UTC"; DAYD:conventions = "Relative julian days with decimal part (as parts of the day)"; DAYD:axis = "t"; DAYD:_FillValue = 99999.; DAYD:epic_code = 601.; | Julian day of the measurement. The integer part represents the day, the decimal part represents the time of the measurement. Date and time are in universal time coordinate. Example : 18833.8013889885 : July 25 2001 19:14:00 |
| LATX | float LATX(DAYD); LATX:long_name = "Latitude of each measurement"; LATX:standard_name = "Latitude"; LATX:units = "degree_north (decimal)"; LATX:_FillValue = 99999.f; LATX:valid_min = -90.f; LATX:valid_max = 90.f; LATX:epic_code = 500; | LATX of the measurement. Unit : degree north Example : 44.4991 for 44° 29’ 56.76’’ N |
| LONX | float LONX (DAYD); LONX:long_name = "Longitude of each measurement"; LONX:standard_name = "Longitude"; LONX:units = "degree_east (decimal)"; LONX:_FillValue = 99999.f; LONX:valid_min = -180.f; LONX:valid_max = 180.f; LONX:epic_code = 501; | LONX of the measurement. Unit : degree east Example : 16.7222 for 16° 43’ 19.92’’ E |
| POSITION_QC | byte POSITION_QC(DAYD) POSITION_QC:long_name=quality flag of position TSG POSITION_QC:standard_name = "Position quality"; POSITION_QC:missing_value=”0”; |  |
| SPDC | float SPDC (DAYD); <PARAM>:long_name = "<Ship speed computed from navigation>"; <PARAM>:units = "<knots>"; <PARAM>:standard_name=”speed” <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <50>; <PARAM>:comment = "<Ship speed from GPS (SOG). If SOG not available, derived from last position>"; <PARAM>:resolution = <> |  |
| REFERENCE_DATE_TIME | char REFERENCE_DATE_TIME(STRING14) REFERENCE_DATE_TIME:comment=”Date of reference for Julian days” REFERENCE_DATE_TIME:conventions=”YYYYMMDDHHMISS” REFERENCE_DATE_TIME:_FillValue=” ”; | Date of reference for julian days The recommended reference data time is “19500101000000” : January 1st 1950 00:00:00 |

### Data series 1 main TSG:

1) Measured variables

| Name | Definition | Comment |
| --- | --- | --- |
| PRES (optional) | float PRES (DAYD); <PARAM>:long_name = "<Sea pressure in TSG>"; <PARAM>:units = "<decibar>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <10>; <PARAM>:comment = "<Sea pressure inside TSG, this is an indication that pump is working properly>"; <PARAM>:resolution = <0.1> | Sea pressure in TSG This is an indication that pump is functioning properly |
| CNDC | float CNDC (DAYD); <PARAM>:long_name = "<Electrical conductivity>"; <PARAM>:units = "<S/m>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <7>; <PARAM>:comment = "<>"; <PARAM>:resolution = <0.001> | Electrical conductivity This data may have been reduced With a median (recommended) or a mean |
| CNDC_STD (optional) | float CNDC_STD(DAYD); <PARAM>:long_name = "<Conductivity standard deviation>"; <PARAM>:units = "<S/m>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = 0; <PARAM>:valid_max = <7>; <PARAM>:comment = "<Standard deviation of conductivity measured by TSG(CNDC)>"; | CNDC, standard deviation for data which have been reduced (with a mean or median) |
| CNDC_CAL (optional) | float CNDC_CAL(DAYD); <PARAM>:long_name = "<Conductivity in TSG – calibrated >"; <PARAM>:units = "<S/m >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <7>; <PARAM>:comment = "< Conductivity, calibrated using linearization coefficients>"; | Conductivity, calibrated using linearization coefficients |
| CNDC_FREQ (optional) | float CNDC_FREQ(DAYD); <PARAM>:long_name = "<Sensor Conductivity Frequency>"; <PARAM>:standard_name = “<Frequency>”; <PARAM>:units = "<Hz >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <20000>; <PARAM>:comment = "< Sensor Conductivity frequency measured by TSG>"; | Sensor Conductivity Frequency |
| SSJT | float SSJT(DAYD); <PARAM>:long_name = "<Temperature within TSG>"; <PARAM>:units = "<degrees Celsius >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "< Temperature within TSG or ‘Jacket Temperature’.>"; <PARAM>:resolution = <> | Temperature within TSG or ‘Jacket Temperature’. Warning: this is not the ocean Temperature It is used to obtain salinity from the conductivity The reduction applied is the same as for conductivity Temperature scale must be specified (T64 or T90) |
| SSJT_STD (optional) | float SSJT_STD(DAYD); <PARAM>:long_name = "<Temperature standard deviation>"; <PARAM>:units = "<Celsius degree >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = <38>; <PARAM>:comment = "< Water jacketTemperature standard deviation inside TSG for data which have been reduced (with a mean or median) >"; | Water jacketTemperature standard deviation inside TSG for data which have been reduced (with a mean or median) |
| SSJT_CAL (optional) | float SSJT_CAL(DAYD); <PARAM>:long_name = "< Water jacket Temperature inside TSG – calibrated >"; <PARAM>:units = "<degrees Celsius >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "< water jacket Temperature inside TSG, calibrated using linearization coefficients>"; | Water jacket Temperature inside TSG, calibrated using linearization coefficients |
| SSJT_FREQ (optional) | float SSJT_FREQ(DAYD); <PARAM>:long_name = "<Water Jacket Sensor Temperature Frequency>"; <PARAM>:standard_name = “<Frequency>”; <PARAM>:units = "<Hz >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <20000>; <PARAM>:comment = "< Sensor temperature frequency measured by TSG>"; |  |
| SSJT_ADJUSTED (optional) | float SSJT_ADJUSTED(DAYD); <PARAM>:long_name = "<Ocean Temperature in TSG adjusted>"; <PARAM>:units = "< deg. Celsius – T90>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "<Y>"; |  |
| SSJT_ADJUSTED_ERROR (optional) | float SSJT_ERR(DAYD); <PARAM>:long_name = "<Error on adjusted water jacket Temperature in TSG>"; <PARAM>:units = "< deg. Celsius >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "< Error on adjusted water jacket Temperature with external data (CTD, XBT, ARGO …)>"; | Error on adjusted water jacket Temperature with external data (CTD, XBT, ARGO …) |
| SSJT_ADJUSTED_QC (optional) | byte SSTP_ADJUSTED_QC(DAYD); <PARAM>_ADJUSTED_QC:long_name = "quality flag"; <PARAM>_ADJUSTED_QC:conventions = "GOSUD ref. table "; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <9>; <PARAM>_QC:_missing_value = 0; | Quality flag applied on adjusted temperature values. The flag scale is specified in table 4. |
| SSJT_ADJUSTED_HIST (optional) | Char SSJT_ADJUSTED_HIST(STRING256) | Adjusted Temperature processing history |

2) Ocean salinity deduced from SSJT and CNDC_TSG

| SSPS | float SSPS(DAYD); <PARAM>:long_name = "<Sea surface practical Salinity (SSTP,CNDC)>"; <PARAM>:units = "<PSU – Sal 78 >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <40>; <PARAM>:comment = "< Sea surface practiicall salinity deduced from conductivity and jacket Temperature>"; | Sea surface practical salinity deduced from conductivity and water jacket Temperature. |
| --- | --- | --- |
| SSPS_QC | byte SSPS_QC(DAYD); <PARAM>_QC:long_name = "quality flag"; <PARAM>_QC:conventions = "GOSUD ref. table "; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <9>; <PARAM>_QC:_missing_value = 0; | Quality flag applied on sea surface practical salinity values. The flag scale is specified in table 4. |
| SSPS_CAL (optional) | float SSPS(DAYD); <PARAM>:long_name = "<Sea Surface Salinity Calibrated (SSTP_CAL,CNDC_CAL)>"; <PARAM>:units = "<PSU – Sal 78 >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <40>; <PARAM>:comment = "<>"; |  |
| SSPS_ADJUSTED (optional) | float SSPS_ADJUSTED(DAYD); <PARAM>:long_name = "<Sea Surface Practical Salinity adjusted>"; <PARAM>:units = "< PSU – Sal 78>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <40>; <PARAM>:comment = "<>"; |  |
| SSPS_ADJUSTED_ERROR (optional) | float SSPS_ADJUSTED_ERR(DAYD); <PARAM>:long_name = "<Error on Sea Surface Practical Salinity adjusted>"; <PARAM>:units = "< PSU – Sal 78>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <40>; <PARAM>:comment = "<>"; |  |
| SSPS_ADJUSTED_QC | byte SSPS_ADJUSTED_QC(DAYD); <PARAM>_ADJUSTED_QC:long_name = "quality flag"; <PARAM>_ADJUSTED_QC:conventions = "GOSUD ref. table "; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <9>; <PARAM>_QC:_missing_value = 0; | Quality flag applied on sea surface practical salinity adjusted values. The flag scale is specified in table 4. |
| SSPS_ADJUSTED_HIST | Char SSPS_ADJUSTED_HIST(STRING256) | Sea Surface Practical Adjusted Salinity processing history |

### Data series 2: ocean  Temperature from the water intake

| Name | Definition | Comment |
| --- | --- | --- |
| SSTP (optional) | float SSTP(DAYD); <PARAM>:long_name = "<Sea Surface Temperature>"; <PARAM>:units = "<deg. Celsius – T90>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "< Sea Surface Temperature measured at intake with Instrument (TINT_TYPE, TINT_NUMBER) This is the ocean Temperature >”; <PARAM>:resolution = <0.001> | Sea Surface Temperature measured at intake with Instrument (TINT_TYPE, TINT_NUMBER) This is the ocean Temperature. |
| SSTP_QC (optional) | byte SSTP_QC(TIME,); <PARAM>_QC:long_name = "quality flag"; <PARAM>_QC:conventions = "GOSUD ref. table "; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <9>; <PARAM>_QC:_missing_value = 0; | Quality flag applied on SSTP values. The flag scale is specified in table 4. |
| SSTP_CAL (optional) | float SSTP_CAL(DAYD); <PARAM>:long_name = "<Sea Surface Temperature Calibrated>"; <PARAM>:units = "< deg. Celsius – T90 >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "<>"; | Sea Surface Temperature Calibrated |
| SSTP_FREQ (optional) | float SSTP_FREQ(DAYD); <PARAM>:long_name = "<Sea Surface Temperature Frequency>"; <PARAM>:standard_name = “<Temperature frequency in TSG>”; <PARAM>:units = "<Hz >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <20000>; <PARAM>:comment = "< Frequency of external temperature sensor, used for SST measurement>"; | Frequency of external temperature sensor, used for SST measurement |
| SSTP_ADJUSTED (optional) | float SSTP_ADJUSTED(DAYD); <PARAM>:long_name = "<Adjusted Sea Surface Temperature adjusted>"; <PARAM>:units = "< deg. Celsius – T90>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "<>"; | Adjusted Sea Surface Temperature |
| SSTP_ADJUSTED_ERROR (optional) | float SSTP_ERR(DAYD); <PARAM>:long_name = "<Error on adjusted Sea Surface Temperature>"; <PARAM>:units = "< deg. Celsius – T90>"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <-1.5>; <PARAM>:valid_max = <38>; <PARAM>:comment = "<>"; | Error on adjusted Sea Surface Temperature |
| SSTP_ADJUSTED_QC (optional) | byte SSTP_ADJUSTED_QC(DAYD); <PARAM>_ADJUSTED_QC:long_name = "quality flag"; <PARAM>_ADJUSTED_QC:conventions = "GOSUD ref. table "; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <9>; <PARAM>_QC:_missing_value = 0; | Quality flag applied on adjusted temperature values. The flag scale is specified in table 4. |
| SSTP_ADJUSTED_HIST | Char SSTP_ADJUSTED_HIST(STRING256) | Adjusted Temperature processing history |

### Data series 3 : external data

| Name | Definition | Comment |
| --- | --- | --- |
| DATE_EXT | char DATE (DAYD_EXT,STRING14); DATE_EXT:long.name = “Date/time of water sample” DATE_EXT:conventions = “yyyymmddhhmmss” | This is the original data describing the date, it must not be lost |
| DAYD_EXT | double DAYD_EXT(DAYD_EXT); DAYD_EXT:long_name = "Julian day (UTC) of each measurement"; DAYD_EXT:standard_name = "time"; DAYD_EXT:units = "days since 1950-01-01 00:00:00 UTC"; DAYD_EXT:conventions = "Relative julian days with decimal part (as parts of the day)"; DAYD_EXT:axis = "t"; DAYD_EXT:_FillValue = 999999.; DAYD_EXT:epic_code = 601.; | Julian day of the measurement. The integer part represents the day, the decimal part represents the time of the measurement. Date and time are in universal time coordinate. Example : 18833.8013889885 : July 25 2001 19:14:00 |
| LATX_EXT | float LATX_EXT(DAYD_WS); LATX_EXT:long_name = "LATX samples"; LATX_EXT:standard_name = "LATX"; LATX_EXT:units = "degree_north"; LATX_EXT:_FillValue = 99999.f; LATX_EXT:valid_min = -90.f; LATX_EXT:valid_max = 90.f; LATX_EXT:epic_code = 500; | LATX of the measurement. Unit : degree north Example : 44.4991 for 44° 29’ 56.76’’ N |
| LONX_EXT | float LONX_EXT(DAYD_EXT); LONX_EXT:long_name = "LONX samples"; LONX_EXT:standard_name = "LONX"; LONX_EXT:units = "degree_east"; LONX_EXT:_FillValue = 99999.f; LONX_EXT:valid_min = -180.f; LONX_EXT:valid_max = 180.f; LONX_EXT:epic_code = 501; | LONX of the measurement. Unit : degree east Example : 16.7222 for 16° 43’ 19.92’’ E |
| SSTP_EXT | float SSTP_EXT (DAYD_EXT); <PARAM>_EXT:long_name = "<Temperature from water samples>"; <PARAM>_EXT:units = "<degrees Celsius >"; <PARAM>:_FillValue = <X>; <PARAM>:valid_min = <Y>; <PARAM>:valid_max = <Y>; <PARAM>:comment = "<Y>"; <PARAM>:resolution = <> | PARAM = SSTP_EXT, |
| SSTP_EXT_QC | byte SSTP_EXT_QC(DAYD_EXT); <PARAM>_EXT_QC:long_name = "quality flag"; <PARAM>_EXT_QC:conventions = " "; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <9>; <PARAM>_QC:_missing_value = 0; | Quality flag applied temperature values. The flag scale is specified in table 4. |
| SSTP_EXT_TYPE | Char:SSPS_EXT_TYPE(STRING4) <SSPS_EXT_TYPE>:long_name = “Type of external data origin” | Example :ARGO, CTD, XBT |
| SSPS_EXT | float SSPS_EXT (DAYD_EXT); <PARAM>_EXT:long_name = "<Sea Surface Salinity from external instrument>"; <PARAM>_EXT:units = "<PSU – Sal 78 >"; <PARAM>:_FillValue = <99999>; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <40>; <PARAM>:comment = "<>"; <PARAM>:resolution = <0.001> | PARAM = SSPS_EXT, |
| SSPS_EXT_QC | byte SSPS_EXT_QC(DAYD_EXT); <PARAM>_EXT_QC:long_name = "quality flag"; <PARAM>_EXT_QC:conventions = "GOSUD table "; <PARAM>:valid_min = <0>; <PARAM>:valid_max = <9>; <PARAM>_QC:_missing_value = 0; | Quality flag applied salinity values. The flag scale is specified in table 4. |
| SSPS_EXT_TYPE | Char:SSPS_EXT_TYPE(DAYD_EXT,STRING4) <SSPS_EXT_TYPE>:long_name = “Type of external data origin” | Example :WS, ARGO, CTD, XBT |
| SSPS_EXT_ANALDATE | char SSPS_EXT_ANALDATE(DAYD_EXT,STRING14); SSPS_EXT_ANALDATE:long_name = "<Date of water sample surface salinity analysis >"; SSPS_WS_ANALDATE:conventions = “yyyymmddhhmiss” |  |
| SSPS_EXT_BOTTLE | char SSPS_EXT_BOTTLE(DAYD_EXT,STRING4); SSPS_EXT_BOTTLE:long_name = “Sea Surface Practical salinity Bottle number”> <SSPS_EXT_BOTTLE>:_FillValue = <0>; | Example; 0001 or B12 |

# Reference tables

## Reference table 1 : Thermosalinometer (TSG) type

Name

SBE21

SBE45

UNKNO

## Reference table 2 : Temperature sensor at intake (TINT) type

Name

SBE38

SBE3S

TQP

UNKNO

NA

## Reference table 3 : PROCESSING STATES

| Code | Libellé |
| --- | --- |
| 0A | RAW DATA |
| 0B | NOT RECOMMEND |
| 0C | NOT RECOMMEND |
| 1A | DATA LOCATED |
| 1B | NOT RECOMMEND |
| 1C | NOT RECOMMEND |
| 2A | NOT RECOMMEND |
| 2B | APPLICATION . D’UN CODE DE QUALITE AUTOMATIQUE |
| 2B+ | APPLICATION D’UN CODE DE QUALITE APRES INSPECTION VISUELLE |
| 2C | DONNEES VALIDEES PAR LE PI |
| 2C+ | CONTROLE CLIMATOLOGIQUE |
| 3A | NON RECOMMANDE |
| 3B | DONNEE REDUITE CALIBREE |
| 3C | DONNEE REDUITE GRILLEE |

## Reference table 4 : QUALITY FLAGS

| n | Meaning |
| --- | --- |
| 0 | No QC was performed |
| 1 | Good data |
| 2 | Probably good date |
| 3 | Bad data that are potentially correctable |
| 4 | Bad data |
| 5 | Value changed |
| 6 | Not used |
| 7 | Not used |
| 8 | Interpolated value |
| 9 | Missing value |

## Reference table 5 : Parameter code table

| Code | Parameter Long name | Unit | Valid min | Valid max | Fortran Format resolution | Fill value |
| --- | --- | --- | --- | --- | --- | --- |
| LATX | Latitude | Decimal degree | -90 | 90 | %+8.41f | 99999 |
| LONX | Longitude | Decimal degree | -180 | 180 | %+9.41f | 99999 |
| DAYD | Decimal Julian day time | Decimal day | 0.0 | 3660.0 | %9.51f | 99999 |
| SPDC | Ship speed computed from navigation | Meter/second | 0 | 90 | %6.31f | 99999 |
| PRES | Sea pressure | decibar | 0 | 6500 | %6.11f | 99999 |
| DEPH | Depth below sea surface | meter | 0 | 6000 | %6.11f | 99999 |
| PSAL | Practical salinity | PSU | 33 | 37 | %6.31f | 99999 |
| CNDC | Electrical conductivity | S/m | 3 | 7 | %5.31f | 99999 |
| SSJT | Sea surface water jacket temperature | Celsius degree | -1.5 | 38 | %6.31f | 99999 |
| SSPS | Sea surface practical salinity | PSU |  |  | %6.31f | 99999 |
| SSTP | Sea surface temperature | Celsius degree | -1.5 | 38 | %6.31f | 99999 |

## Reference table 6 : Water sample (bottle) type

Name

OSIL

UNKNO

NA