Gosud

GOSUD

DATA FORMAT TSG

Version 1.6

January 12th 2010

Table of content

History	4

1.	Overview of the format	5

2.	Description of the thermosalinometer format	6

2.1.	Data file dimensions	6

2.2.	Global attributes: meta-data	7

2.3.	Measurements	9

2.3.1.	Variables describing installation of TSG	9

2.3.2.	Variables describing installation of Temperature sensor at intake (TINT)	10

2.3.3.	Coordinates for TSG/TINT:	11

2.3.4.	Data series 1 main TSG:	13

2.3.5.	Data series 2: Ocean Temperature from the water intake	16

2.3.6.	Data series 3 : external data	17

3.	Reference tables	19

3.1.	Reference table 1 : Thermosalinometer (TSG) type	19

3.2.	Reference table 2 : Temperature sensor at intake (TINT) type	19

3.3.	Reference table 3 : PROCESSING STATES	19

3.4.	Reference table 4 : QUALITY FLAGS	20

3.5.	Reference table 5 : Parameter code table	20

3.6.	Reference table 6 : Water sample (bottle) type	21

3.7.	Reference table 7 : Data centres and institutions codes	21

History

| Version | Date | Comment |
| --- | --- | --- |
| 1.0 | 11/09/2005 | F. Gaillard/L. Petit de la Villéon creation of the document based on GOSUD data management user’s manual (T. Carval) |
| 1.1 | 11/29/2005 | FG/LPdV: revision after ORE-SSS technical meeting |
| 1.2 | 12/01/2005 | T. Carval : parameter naming conventions : <PARAM>_XXX where XXX = TSG, WS, TINT |
| 1.3 | 01/17/2006 | F. Gaillard, D. Mathias, correction after producing the 2000-2004 files. |
| 1.4 | 07/07/2008 | C. Lagadec, J.Grelet, F. Gaillard : integration in processing CORIOLIS/IRD |
| 1.5 | 24/02/2009 | J.Grelet: add data centres table add dimension STRING8 remove dimension DAYD_WS and change NOCOEF_CAL to 6 in § 2.1 correct dimension for SSTP_QC in §2.3.5 and LATX_EXT § 2.3.6 add new variables _CONV for calibration coefficients used to store coefficients names |
| 1.6 | 12/17/2009 01/12/2010 | J.Grelet: use <PARAM> for all description of variables attribute and remove <> when field is not a template Change attribute missing_value with default_value Add global attribute DATE_TINT, CONVENTIONS and TYPE_POSITION And some corrections to agree NetCDF file, add variable attributes standard_name and coordinate Set date with English convention in history Remove FillValue for <PARAM>_CALCOEFF_CONV, <PARAM>_LINCOEFF_CONV and SSPS_EXT_BOTTLE |

# Overview of the format

This document aims at defining a common format for thermo-salinometer (TSG) data. This format is meant for the data exchange and processing. It should hold both real time and delayed mode data and various levels of resolution or processing.

The data sets found in the file are:

- General information on the file, the platform and the TSG installation

- Data series from the main TSG

And, whenever available:

- Data series from the Temperature sensor at water intake

- Data from salinity sample analysis

In the case of real time data, some variables may be missing. The file is updated as new data and information come in.

For delayed mode use, the data center is expected to provide a file that corresponds to a unique installation period of a thermo-salinometer on one ship. All available information must have been entered, in particular those referring to the calibration and water sample analysis. The group in charge of the validation will complement the file with the ‘corrected’ data series.

# Description of the thermosalinometer format

## Data file dimensions

| Name | Definition | Comment |
| --- | --- | --- |
| DAYD |  | Number of recorded measurements for the main TSG series |
| DAYD_EXT |  | Number of recorded measurements for external data |
| NCOEF_CAL | NCOEF_CAL = 7 | Number of calibration coefficients |
| NCOEF_LIN | NCOEF_LIN = 2 | Number of drift correction coefficients (linear) |
| STRING256 STRING14 STRING8 STRING4 | STRING256 = 256; STRING14 = 14; STRING8 = 8; STRING4 = 4; | String dimensions. |
| N1 | N1 = 1; |  |

## Global attributes: meta-data

| Name | Definition | Comment |
| --- | --- | --- |
| CYCLE_MESURE | CYCLE_MESURE = <char value>; | Cruise name or travel number Example : EGEE6 or PAST0601 |
| PROJECT_NAME | PROJECT_NAME = <char value>; | Name of the project which operates the TSG line. Example : ORE-SSS |
| PLATFORM_NAME | PLATFORM_NAME = <char value>; | Ship name Example : Pourquoi Pas ? |
| SHIP_CALL_SIGN | SHIP_CALL_SIGN = <char value>; | Ship call sign Example : FABB |
| SHIP_MMSI | SHIP_CALL_MMSI = <char value>; | Ship MMSI (ASN) number Example : 227 222 00 |
| DATE_TSG | DATE_TSG = <char value>; | Date of TSG installation : YYYYMMDDHHMISS |
| TYPE_TSG | TYPE_TSG = <char value>; | Described in reference table 1 Example : SBE21 |
| NUMBER_TSG | NUMBER_TSG= <char value>; | (serial number, ex: 2250) |
| DATE_TINT | DATE_TINT = <char value> | Date of TINT installation |
| TYPE_TINT | TYPE_TINT= <char value>; | Temperature sensor at intake. Described in reference table 2 Ex: SBE3 |
| NUMBER_TINT | NUMBER_TINT= <char value>; | (serial number, ex: ????) |
| DATA_TYPE | DATA_TYPE = <char value>; | This field describes the type of data contained in the file. Example : TRAJECTORY, PROFIL or TIME_SERIE |
| DATA_MODE | DATA_MODE = <char value>; | Indicates if the file contains real time or delayed mode data. R : real time data D : delayed mode data |
| SAMPLING_PERIOD | SAMPLING_PERIOD = <char value>; | Sampling period in seconds: 6 to 3600 |
| DATE_START | DATE_START = <char value>; | Date of first measurements : YYYYMMDDHHMISS |
| DATE_END | DATE_END = <char value> | Date of last measurements : YYYYMMDDHHMISS |
| SOUTH_LATX |  | South limit of measurements |
| NORTH_LATX |  | North limit of measurements |
| WEST_LONX |  | West limit of measurements |
| EAST_LONX |  | East limit of measurements |
| FORMAT_VERSION | FORMAT_VERSION = <char value>; | File format version : 1.6 for this format |
| CONVENTIONS | CONVENTIONS = = <char value>; | GOSUB 1.6, CF1.4 |
| DATE_CREATION | DATE_CREATION = <char value>; | Date and time (UTC) of creation of this file. Format : YYYYMMDDHHMISS Example : 20011229161700 : December 29th 2001 16:17:00 |
| DATE_UPDATE (optional) | DATE_UPDATE = <char value>; | Date and time (UTC) of update of this file. Format : YYYYMMDDHHMISS Example : 20011230161700 : December 30th 2001 16:17:00 |
| DATA_RESTRICTIONS | DATA_RESTRICTIONS = <char value>; | Restriction on use for these data. Example : "NONE" |
| CITATION | CITATION = <char value>; | The citation should be used for publications. Example : "These data were collected and made freely available by the International Gosud Project and the national programmes that contribute to it." |
| COMMENT | COMMENT = <char value>; |  |
| PI_NAME | PI_NAME = <char value>; | Name of the principal investigator in charge of the TSG line. Example : GENAVIR |
| DATA_CENTRE | DATA_CENTRE = <char value>; | Code for the data centre (2 char) The data centre codes are described in the reference table 7 Example : IF for Ifremer, France |
| DATA_ACQUISITION | DATA_ACQUISITION = <char value>; | Acquisition data centre Example : SHOM, IRD, GENAVIR, CNRS … |
| PROCESSING_CENTRE | PROCESSING_CENTRE = <char value>; | Processing data centre Example : ORE-SSS, CORIOLIS/IRD, CORIOLIS/SISMER |
| PROCESSING_STATES | PROCESSING_STATES = <char value>; | Described in reference table 3 |
| WS_TYPE | WS _TYPE = <char value> | Model of water sample bottle Default : NA Example: OSIL Described in reference table 6 |
| TYPE_POSITION | TYPE_POSITION = <char value>; | Example: GPS, GPS DIFFERENTIAL, ARGOS, INTERPOLATE, MANUAL, NONE, UNKNOW |

## Measurements

### Variables describing installation of TSG

| Name | Definition | Comment |
| --- | --- | --- |
| SSPS_DEPH | float SSPS_DEPH (N1); <PARAM>:long_name = "Nominal depth of water intake for salinity measurement"; <PARAM>:units = "meter"; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 100.; <PARAM>:format = “%6.3f”; <PARAM>:_FillValue = 99999.f; | Ex: SSPS_DEPH = 8.0 |
| SSPS_DEPH_MIN | float SSPS_DEPH_MIN (N1); <PARAM>:long_name = "Minimum depth of water intake"; <PARAM>:units = "meter"; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 100.; <PARAM>:format = “%6.3f”; <PARAM>:_FillValue = 99999.f; | Ex: SSPS_DEPH_MIN = 6.0 |
| SSPS_DEPH_MAX | float SSPS_DEPH_MAX (N1); <PARAM>:long_name = "Maximum depth of water intake"; <PARAM>:units = "meter"; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 100.; <PARAM>:format = “%6.3f”; <PARAM>:_FillValue = 99999.f; | Ex: SSPS_DEPH_MAX = 10.0 |
| CNDC_CALCOEF | double CNDC_CALCOEF(NCOEF_CAL); <PARAM>:long_name = "Conductivity calibration coefficients"; <PARAM>_FillValue = 99999.f; | Calibration coefficients are real value date is in julian day from REFERENCE_DATE_TIME |
| CNDC_CALCOEF_CONV | char CNDC_CALCOEF_CONV(NCOEF_CAL, STRING8); <PARAM>:long_name = "Conductivity calibration coefficients convention"; | Calibration coefficient convention is an enumeration of coefficients name Ex: “ ‘date’, ‘a’,’b’,’c’,’d’,’m’” or “ ‘date’,’g’,’h’,’I’,’j’,’cpcor’,’ctcor’” |
| CNDC_LINCOEF | double CNDC_LINCOEF(NCOEF_LIN); <PARAM>:long_name = "Conductivity linear drift correction coefficients"; <PARAM>_FillValue = 99999.f; | Calibration coefficients are real value date is in julian day from REFERENCE_DATE_TIME |
| CNDC_LINCOEF_CONV | char CNDC_LINCOEF_CONV(NCOEF_LIN, STRING8); <PARAM>:long_name = "Conductivity linear drift correction coefficients convention "; | Calibration coefficient convention is an enumeration of coefficients name Ex: " ‘date’, ‘slope’, ‘offset’ " |
| SSJT_CALCOEF | double SSJT_CALCOEF(NCOEF_CAL); <PARAM> :long_name = "Temperature calibration coefficients"; <PARAM> :_FillValue = 99999.lf; | Calibration coefficients are real value date is in julian day from REFERENCE_DATE_TIME |
| SSJT_CALCOEF_CONV | char SSJT_CALCOEF_CONV(NCOEF_CAL, STRING8); <PARAM>:long_name = " Temperature calibration coefficients convention"; | Calibration coefficient convention is an enumeration of coefficients name Ex: “ ‘date’, ‘a’,’b’,’c’,’d’,’f0’” or “ ‘date’,’g’,’h’,’I’,’j’,’f0’” |
| SSJT_LINCOEF | double SSJT_LINCOEF(NCOEF_LIN); <PARAM> :long_name = "Temperature linear drift correction coefficients"; <PARAM> _FillValue = 99999.lf; | Calibration coefficients are real value date is in julian day from REFERENCE_DATE_TIME |
| SSJT_LINCOEF_CONV | char SSJT_LINCOEF_CONV (NCOEF_LIN, STRING8); <PARAM> :long_name = "Temperature linear drift correction coefficients"; | Calibration coefficient convention is an enumeration of coefficients name Ex: " ‘date’,’slope’, ‘offset’ " |

### Variables describing installation of Temperature sensor at intake (TINT)

| Name | Definition | Comment |
| --- | --- | --- |
| SSTP_DEPH | float SSTP_DEPH (N1); <PARAM>:long_name = "Nominal depth of water intake for Temperature measurement"; <PARAM>:units = "meter"; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 100.; <PARAM>:format = “%6.3f”; <PARAM>:_FillValue = 99999.f; | Ex: SSTP_DEPH = 8.0 |
| SSTP_DEPH_MIN | float SSTP_DEPH_MIN (N1); <PARAM>:long_name = "Minimum depth of water intake for Temperature measurement "; <PARAM>:units = "meter"; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 100.; <PARAM>:format = “%6.3f”; <PARAM>:_FillValue = 99999.f; | Ex: SSTP_DEPH_MIN = 6.0 |
| SSTP_DEPH_MAX | float SSTP_DEPH_MAX (N1); <PARAM>:long_name = "Maximum depth of water intake for Temperature measurement "; <PARAM>:units = "meter"; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 100.; <PARAM>:format = “%6.3f”; <PARAM>:_FillValue = 99999.f; | Ex: SSTP_DEPH_MAX = 10.0 |
| SSTP_CALCOEF | double SSTP_CALCOEF(NCOEF_CAL); <PARAM>:long_name = "Temperature calibration coefficients"; <PARAM>:_FillValue = 99999.lf; | Calibration coefficients are real value date is in julian day from REFERENCE_DATE_TIME |
| SSTP_CALCOEF_CONV | char SSTP_CALCOEF_CONV(NCOEF_CAL,STRING8); <PARAM>: long_name = " Temperature calibration coefficients convention"; | Calibration coefficient convention is an enumeration of coefficients name Ex: “ ‘date’, ‘a’,’b’,’c’,’d’,’f0’” Or “ ‘date’,’g’,’h’,’I’,’j’,’f0’” |
| SSTP_LINCOEF | double SSTP_LINCOEF(NCOEF_LIN); <PARAM> :long_name = "Temperature linear drift correction coefficients"; <PARAM> :_FillValue = 99999.lf; | Calibration coefficients are real value date is in julian day from REFERENCE_DATE_TIME |
| SSTP_LINCOEF_CONV | char SSTP_LINCOEF_CONV (NCOEF_LIN,STRING8); <PARAM> :long_name = "Temperature linear drift correction coefficients"; | Calibration coefficient convention is an enumeration of coefficients name Ex: " ‘date’,’slope’, ‘offset’ " |

### Coordinates for TSG/TINT:

| Name | Definition | Comment |
| --- | --- | --- |
| DATE | char DATE (DAYD,STRING14); <PARAM>:long_name = “Date of main instrument measurement”; <PARAM>:conventions = “yyyymmddhhmmss”; <PARAM>:coordinate = “DAYD”; | This is the original data describing the date, it must not be lost |
| DAYD | double DAYD(DAYD); <PARAM>:long_name = "Decimal julian day (UTC) of each measurement"; <PARAM>:standard_name = "time"; <PARAM>:units = "days since 1950-01-01 00:00:00 UTC"; <PARAM>:conventions = "Relative julian days with decimal part (as parts of the day)"; <PARAM>:_FillValue = 99999.lf; <PARAM>:valid_min = 0..; <PARAM>:valid_max = 36600.; <PARAM>:format = “%11.5f”; <PARAM>:axis = "T"; <PARAM>:epic_code = 601; <PARAM>:coordinate = “DAYD”; | Julian day of the measurement. The integer part represents the day, the decimal part represents the time of the measurement. Date and time are in universal time coordinate. Example : 18833.80140 : July 25 2001 19:14:00 |
| LATX | float LATX(DAYD); <PARAM>:long_name = "Latitude of each measurement"; <PARAM>:standard_name = "latitude"; <PARAM>:units = "degree_north"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -90.; <PARAM>:valid_max = 90.; <PARAM>:format = “%8.4f”; <PARAM>:axis = "Y"; <PARAM>:epic_code = 500; <PARAM>:coordinate = “DAYD”; | Latitude of the measurement. Example : 44.4991 for 44° 29’ 56.76’’ N |
| LONX | float LONX (DAYD); <PARAM>:long_name = "Longitude of each measurement"; <PARAM>:standard_name = "longitude"; <PARAM>:units = "degree_east"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -180.; <PARAM>:valid_max = 180.; <PARAM>:format = “%9.4f”; <PARAM>:axis = "X"; <PARAM>:epic_code = 501; <PARAM>:coordinate = “DAYD”; | Longitude of the measurement. Example : 16.7222 for 16° 43’ 19.92’’ E |
| POSITION_QC | byte POSITION_QC(DAYD) <PARAM>:long_name=quality flag of position TSG <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD”; | If variable not exist, it is set at default_value during creation. |
| SPDC | float SPDC (DAYD); <PARAM>:long_name = "Ship speed computed from navigation"; <PARAM>:units = "knots"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 50.; <PARAM>:format = “%6.1f”; <PARAM>:resolution = 0.1; <PARAM>:coordinate = “DAYD”; | Ship speed from GPS (SOG - Speed Over Ground). If SOG not available, derived from last position |
| REFERENCE_DATE_TIME | char REFERENCE_DATE_TIME(STRING14) <PARAM>:conventions=”YYYYMMDDHHMISS” | Reference date for julian days origin The recommended reference data time is “19500101000000” : January 1st 1950 00:00:00 |

### Data series 1 main TSG:

1) Measured variables

| Name | Definition | Comment |
| --- | --- | --- |
| PRES (optional) | float PRES (DAYD); <PARAM>:long_name = "Sea pressure in TSG"; <PARAM>:standard_name = "sea_water_pressure"; <PARAM>:units = "decibar=1000 pascals"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 10.; <PARAM>:resolution = 0.1; <PARAM>:format = “%6.1f”; <PARAM>:coordinate = “DAYD”; | Sea pressure in TSG, this is an indication that pump is functioning properly |
| FLOW (optional) | float FLOW (DAYD); <PARAM>:long_name = "Measuring pump discharge in the TSG inlet"; <PARAM>:units = "l/min"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 100.; <PARAM>:resolution = 0.1; <PARAM>:format = “%6.1f”; <PARAM>:coordinate = “DAYD”; | Water pump discharge measurement in the TSG inlet, this is an indication that pump is working properly |
| CNDC | float CNDC (DAYD); <PARAM>:long_name = "Electrical conductivity"; <PARAM>:units = "S/meter"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 7.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Conductivity measured by TSG. This data may have been reduced with a median (recommended) or a mean |
| CNDC_STD (optional) | float CNDC_STD(DAYD); <PARAM>:long_name = "Conductivity standard deviation"; <PARAM>:units = "S/m"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 7.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Standard deviation of conductivity measured by TSG for data which have been reduced (with a mean or median) |
| CNDC_CAL (optional) | float CNDC_CAL(DAYD); <PARAM>:long_name = "Conductivity calibrated "; <PARAM>:units = "S/meter"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 7.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Conductivity, calibrated using linearization coefficients |
| CNDC_FREQ (optional) | float CNDC_FREQ(DAYD); <PARAM>:long_name = "Sensor Conductivity Frequency"; <PARAM>:units = "Hz"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 20000.; <PARAM>:resolution = 0.001; <PARAM>:format = “%9.3f”; <PARAM>:coordinate = “DAYD”; | Sensor Conductivity Frequency measured by TSG |
| SSJT | float SSJT(DAYD); <PARAM>:long_name = "Water jacket temperature"; <PARAM>:units = "degree_Celsius"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Temperature within TSG or water jacket temperature. Warning: this is not the ocean temperature (SST) It is used to obtain salinity from the conductivity The reduction applied is the same as for conductivity. Temperature scale is ITS-90 |
| SSJT_QC | byte SSTP_QC(DAYD); <PARAM>:long_name = "Water jacket temperature quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD”; | Quality flag applied on water jacket temperature values. |
| SSJT_STD (optional) | float SSJT_STD(DAYD); <PARAM>:long_name = "Temperature standard deviation"; <PARAM>:units = "degree_Celsius"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Water jacket temperature standard deviation for data which have been reduced (with a mean or median) |
| SSJT_CAL (optional) | float SSJT_CAL(DAYD); <PARAM>:long_name = "Water jacket temperature calibrated"; <PARAM>:units = "degree_Celsius"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Water jacket temperature calibrated using linearization coefficients |
| SSJT_FREQ (optional) | float SSJT_FREQ(DAYD); <PARAM>:long_name = "Water jacket sensor temperature frequency"; <PARAM>:units = "Hz"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 20000.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Frequency of water jacket temperature sensor |
| SSJT_ADJUSTED (optional) | float SSJT_ADJUSTED(DAYD); <PARAM>:long_name = " Water jacket temperature adjusted"; <PARAM>:units = "degree_Celsius"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Adjusted water jacket temperature with external data (ARGO, CTD, XBT,...) |
| SSJT_ADJUSTED_ERROR (optional) | float SSJT_ERR(DAYD); <PARAM>:long_name = "Error on adjusted water jacket Temperature"; <PARAM>:units = "degree_Celsius"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Error on adjusted water jacket temperature with external data (ARGO, CTD, XBT,…) |
| SSJT_ADJUSTED_QC (optional) | byte SSJT_ADJUSTED_QC(DAYD); <PARAM>:long_name = "quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD”; | Quality flag applied on adjusted water jacket temperature values. The flag scale is specified in table 4. If variable not exist, it is set at default_value during creation |
| SSJT_ADJUSTED_HIST (optional) | Char SSJT_ADJUSTED_HIST(STRING256) | Water jacket temperature inside TSG adjusted processing history |

2) Ocean salinity deduced from SSJT and CNDC_TSG

| SSPS | float SSPS(DAYD); <PARAM>:long_name = "Sea surface salinity"; <PARAM>:standard_name = "sea_surface_salinity"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 40.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Sea Surface Salinity (SSS) deduced from conductivity and water jacket temperature. This is the Ocean surface salinity. |
| --- | --- | --- |
| SSPS_QC | byte SSPS_QC(DAYD); <PARAM>:long_name = "Sea surface salinity quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD”; | Quality flag applied on sea surface salinity values. The flag scale is specified in table 4. If variable not exist, it is set at default_value during creation |
| SSPS_CAL (optional) | float SSPS(DAYD); <PARAM>:long_name = "Sea Surface Salinity Scale Calibrated (SSTP_CAL,CNDC_CAL)"; <PARAM>:standard_name = "sea_surface_salinity"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 40.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Sea surface calibrated salinity deduced from conductivity and water jacket temperature |
| SSPS_ADJUSTED (optional) | float SSPS_ADJUSTED(DAYD); <PARAM>:long_name = "Sea surface salinity adjusted"; <PARAM>:standard_name = "sea_surface_salinity"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 40.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Sea surface salinity adjusted with external data (ARGO, CTD, XBT,...) |
| SSPS_ADJUSTED_ERROR (optional) | float SSPS_ADJUSTED_ERR(DAYD); <PARAM>:long_name = "Error on sea surface salinity adjusted"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 40.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Error on sea surface salinity adjusted with external data (ARGO CTD, XBT,...) |
| SSPS_ADJUSTED_QC (optional) | byte SSPS_ADJUSTED_QC(DAYD); <PARAM>:long_name = "Sea surface salinity adjusted quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD”; | Quality flag applied on adjusted sea surface salinity The flag scale is specified in table 4. If variable not exist, it is set at default_value during creation |
| SSPS_ADJUSTED_HIST | char SSPS_ADJUSTED_HIST(STRING256) | Sea surface adjusted salinity processing history |

### Data series 2: Ocean Temperature from the water intake

| Name | Definition | Comment |
| --- | --- | --- |
| SSTP (optional) | float SSTP(DAYD); <PARAM>:long_name = "Sea surface temperature"; <PARAM>:standard_name = “sea_surface_temperature"; <PARAM>:units = "degree_Celsius"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Sea Surface Temperature (SST) measured at intake with external instrument (TINT_TYPE, TINT_NUMBER) This is the Ocean surface temperature. |
| SSTP_QC (optional) | byte SSTP_QC(DAYD); <PARAM>:long_name = " Sea surface temperature quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD”; | Quality flag applied on SSTP values. The flag scale is specified in table 4. If variable not exist, it is set at default_value during creation |
| SSTP_CAL (optional) | float SSTP_CAL(DAYD); <PARAM>:long_name = "Sea surface temperature Calibrated"; <PARAM>:units = "degree_Celsius”; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Sea surface temperature calibrated |
| SSTP_FREQ (optional) | float SSTP_FREQ(DAYD); <PARAM>:long_name = "Sea surface temperature frequency"; <PARAM>:units = "Hz"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 20000.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Frequency of external temperature sensor, used for SST measurement |
| SSTP_ADJUSTED (optional) | float SSTP_ADJUSTED(DAYD); <PARAM>:long_name = "Adjusted sea surface temperature adjusted"; <PARAM>:units = «degree_Celsius»; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Adjusted sea surface temperature |
| SSTP_ADJUSTED_ERROR (optional) | float SSTP_ERR(DAYD); <PARAM>:long_name = "Error on adjusted sea surface temperature"; <PARAM>:units = «degree_Celsius»; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD”; | Error on adjusted sea surface temperature |
| SSTP_ADJUSTED_QC (optional) | byte SSTP_ADJUSTED_QC(DAYD); <PARAM>:long_name = " Sea Surface Temperature quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD”; | Quality flag applied on adjusted temperature values. The flag scale is specified in table 4. If variable not exist, it is set at default_value during creation |
| SSTP_ADJUSTED_HIST | Char SSTP_ADJUSTED_HIST(STRING256) | Adjusted Temperature processing history |

### Data series 3 : external data

| Name | Definition | Comment |
| --- | --- | --- |
| DATE_EXT | char DATE_EXT (DAYD_EXT,STRING14); <PARAM>:long.name = “Date of each external data measurement” <PARAM>:conventions = “yyyymmddhhmmss” | This is the original data describing the date, it must not be lost |
| DAYD_EXT | double DAYD_EXT(DAYD_EXT); <PARAM>:long_name = "Decimal julian day (UTC) of external data measurement"; <PARAM>:standard_name = "time"; <PARAM>:units = "days since 1950-01-01 00:00:00 UTC"; <PARAM>:conventions = "Relative julian days with decimal part (as parts of the day)"; <PARAM>:_FillValue = 99999.lf; <PARAM>:valid_min = 0..; <PARAM>:valid_max = 36600.; <PARAM>:format = “%11.5f”; <PARAM>:axis = "T"; <PARAM>:epic_code = 601.; <PARAM>:coordinate = “DAYD_EXT”; | Julian day of the measurement. The integer part represents the day, the decimal part represents the time of the measurement. Date and time are in universal time coordinate. Example : 18833.80140 : July 25 2001 19:14:00 |
| LATX_EXT | float LATX_EXT(DAYD_EXT); <PARAM>:long_name = "Latitude for each external data measurement"; <PARAM>:standard_name = "latitude"; <PARAM>:units = "degree_north"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -90.; <PARAM>:valid_max = 90.; <PARAM>:format = “%8.4f”; <PARAM>:axis = "Y"; <PARAM>:epic_code = 500; <PARAM>:coordinate = “DAYD_EXT”; | Latitude of the measurement (decimal). Example : 44.4991 for 44° 29’ 56 N |
| LONX_EXT | float LONX_EXT(DAYD_EXT); <PARAM>:long_name = " Longitude for each external data measurement "; <PARAM>:standard_name = "longitude"; <PARAM>:units = "degree_east"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -180.; <PARAM>:valid_max = 180.; <PARAM>:format = “%9.4f”; <PARAM>:axis = "X"; <PARAM>:epic_code = 501; <PARAM>:coordinate = “DAYD_EXT”; | Longitude of the measurement (decimal. Example : 16.7222 for 16° 43’ 19 E |
| SSTP_EXT | float SSTP_EXT (DAYD_EXT); <PARAM>:long_name = "Sea surface temperature from external data"; <PARAM>:standard_name = “sea_surface_temperature"; <PARAM>:units = "degree_Celsius"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = -1.5; <PARAM>:valid_max = 38.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD_EXT”; | Sea Surface Temperature (SST) from external data instrument (ARGO, CTD, XBT) |
| SSTP_EXT_QC | byte SSTP_EXT_QC(DAYD_EXT); <PARAM>:long_name = " Sea surface temperature from external data quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:_default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD_EXT”; | Quality flag applied on external sea surface temperature data values. The flag scale is specified in table 4. If variable not exist, it is set at default_value during creation |
| SSTP_EXT_TYPE | Char:SSPS_EXT_TYPE(DAYD_EXT,STRING4) <PARAM>:long_name = “Type of sea surface temperature external data origin”; <PARAM>:coordinate = “DAYD_EXT”; | Example :ARGO, CTD, XBT |
| SSPS_EXT | float SSPS_EXT (DAYD_EXT); <PARAM>:long_name = "Sea surface salinity from external data"; <PARAM>:standard_name = "sea_surface_salinity"; <PARAM>:_FillValue = 99999.f; <PARAM>:valid_min = 0.; <PARAM>:valid_max = 40.; <PARAM>:resolution = 0.001; <PARAM>:format = “%6.3f”; <PARAM>:coordinate = “DAYD_EXT”; | Sea surface salinity from external data – water samples included |
| SSPS_EXT_QC | byte SSPS_EXT_QC(DAYD_EXT); <PARAM>:long_name = " Sea surface salinity from external data quality flag"; <PARAM>:valid_min = 0; <PARAM>:valid_max = 9; <PARAM>:default_value = 0; <PARAM>:format = “%1d”; <PARAM>:coordinate = “DAYD_EXT”; | Quality flag applied on external sea surface salinity values data values. The flag scale is specified in table 4. If variable not exist, it is set at default_value during creation |
| SSPS_EXT_TYPE | Char:SSPS_EXT_TYPE(DAYD_EXT,STRING4) <PARAM>:long_name = “Type of external sea surface salinity data origin”; <PARAM>:coordinate = “DAYD_EXT”; | Example :WS (Water Sample), ARGO, CTD, XBT |
| SSPS_EXT_ANALDATE | char SSPS_EXT_ANALDATE(DAYD_EXT,STRING14); <PARAM>:long_name = "Date of water sample surface salinity analysis"; <PARAM>:conventions = “yyyymmddhhmiss” <PARAM>:coordinate = “DAYD_EXT”; | Date of sea surface salinity water sample analysis |
| SSPS_EXT_BOTTLE | char SSPS_EXT_BOTTLE(DAYD_EXT,STRING4); <PARAM>:long_name = “Sea surface salinity bottle number” <PARAM>:coordinate = “DAYD_EXT”; | Number of sea surface salinity water sample Example; 0001 or B12 |

# Reference tables

## Reference table 1 : Thermosalinometer (TSG) type

Name

SBE21

SBE45

UNKNO

## Reference table 2 : Temperature sensor at intake (TINT) type

Name

SBE38

SBE3S

TQP

UNKNO

NA

## Reference table 3 : PROCESSING STATES

| Code | Libellé |
| --- | --- |
| 0A | RAW DATA |
| 0B | NOT RECOMMEND |
| 0C | NOT RECOMMEND |
| 1A | DATA LOCATED |
| 1B | NOT RECOMMEND |
| 1C | NOT RECOMMEND |
| 2A | NOT RECOMMEND |
| 2B | APPLICATION . D’UN CODE DE QUALITE AUTOMATIQUE |
| 2B+ | APPLICATION D’UN CODE DE QUALITE APRES INSPECTION VISUELLE |
| 2C | DONNEES VALIDEES PAR LE PI |
| 2C+ | CONTROLE CLIMATOLOGIQUE |
| 3A | NON RECOMMANDE |
| 3B | DONNEE REDUITE CALIBREE |
| 3C | DONNEE REDUITE GRILLEE |

## Reference table 4 : QUALITY FLAGS

| n | Meaning |
| --- | --- |
| 0 | No QC was performed |
| 1 | Good data |
| 2 | Probably good date |
| 3 | Bad data that are potentially correctable |
| 4 | Bad data |
| 5 | Value changed |
| 6 | Not used |
| 7 | Not used |
| 8 | Interpolated value |
| 9 | Missing value |

## Reference table 5 : Parameter code table

| Code | Parameter Long name | Unit | Valid min | Valid max | Fortran Format resolution | Fill value |
| --- | --- | --- | --- | --- | --- | --- |
| LATX | Latitude | Decimal degree | -90 | 90 | %+8.41f | 99999 |
| LONX | Longitude | Decimal degree | -180 | 180 | %+9.41f | 99999 |
| DAYD | Decimal Julian day time | Decimal day | 0.0 | 3660.0 | %9.51f | 99999 |
| SPDC | Ship speed computed from navigation | Meter/second | 0 | 90 | %6.31f | 99999 |
| PRES | Sea pressure | decibar | 0 | 6500 | %6.11f | 99999 |
| DEPH | Depth below sea surface | meter | 0 | 6000 | %6.11f | 99999 |
| PSAL | Practical salinity | PSU | 33 | 37 | %6.31f | 99999 |
| CNDC | Electrical conductivity | S/m | 3 | 7 | %5.31f | 99999 |
| SSJT | Sea surface water jacket temperature | Celsius degree | -1.5 | 38 | %6.31f | 99999 |
| SSPS | Sea surface practical salinity | PSU |  |  | %6.31f | 99999 |
| SSTP | Sea surface temperature | Celsius degree | -1.5 | 38 | %6.31f | 99999 |

## Reference table 6 : Water sample (bottle) type

Name

OSIL

UNKNO

NA

## Reference table 7 : Data centres and institutions codes

| Code | Description |
| --- | --- |
| AO | AOML, USA |
| BO | BODC, United Kingdom |
| CI | Institute of Ocean Sciences, Canada |
| CS | CSIRO, Australia |
| GE | BSH, Germany |
| GT | GTS : used for data coming from WMO GTS network |
| HZ | CSIO, China Second Institute of Oceanography |
| IF | Ifremer, France |
| IN | INCOIS, India |
| JA | JMA, Japan |
| JM | Jamstec, Japan |
| KM | KMA, Korea |
| ME | MEDS, Canada |
| NA | NAVO, USA |
| PM | PMEL, USA |
| RU | Russia |
| SI | SIO, Scripps, USA potentially correctable |
| SP | Spain |
| UW | University of Washington, USA |
| IR | IRD, France |